| Date | Card PNR | Firmware version / PNR | Change | Description |
| --- | --- | --- | --- | --- |
| January 2018 | 200-595-081-xxx | 81 / 209-595-400-081 (PFM-541I) and 209-595-300-081 (MSM586) | New feature | System: Added support for the PROFINET fieldbus communications protocol in order to allow the latest CPUM cards to replace earlier CPUM cards (PNR 200-595-801-xxx) running a version of firmware that supports PROFINET (as these earlier CPUM cards that support PROFINET are no longer available). Accordingly, PROFINET is now effectively available to all customers. See PCR 17402 for further information. |
|  |  |  | Bug fix (Ref: D#10570) | Fixed the PROFINET stack bug that led to missing data. |
|  |  |  | Compatibility | This firmware release (081) is compatible with all possible hardware configurations of the CPUM card using the PFM-541I or MSM586 CPU module. (This version does not support the older MSM486 CPU module.) For information on CPUM card firmware and VM600 MPS software compatibility, refer to the latest version of the VM600 MPS software release notes. Note: This version is based on the 080 version of firmware and adds the PROFINET protocol which was available in the 801 version of firmware (no longer supported). |